THE INITIAL SET UP FOR THE MAIN BOARD OF LENS EDGER

- CLEANING THE MEMORY

1.Press“R” and switch on the power.

2.Press “STOP” and “Du Du” sound would be heard

3.Switch off the power.

- SETTING UP THE LAMP

1.Press”FRAME” and switch on the power.”Du Du”sound would be heard

2 Adjust “GL” LAMP to be light

3.Press”STOP” and “Du Du” sound would be heard.

4.Switch off the power

C.SIZE ADJUSTMENT

1.Press “+” and switch on the power,”Du Du”sound would be heard.

2.Use “-“or “+”button to adjust until  +0.00.

3.Press”STOP”and “Du Du”sound would be heard

4. Press”FRAME” until”P”is light.

5.Use “-“to adjust to -0.10

6.Press”STOP”and “Du Du”sound would be heard.

7.Switch off the power

D.CIRCLES SETTING UP.

1.Press “LENS” and switch on the power until “CR”lamp is light

2.Adjust the data to +2 by pressing”+”or”-“button

3Press “STOP” and “Du Du” sound would be heard

3.Press:LENS” to “GL” lamp is light

5.Adjust the data to +2 by pressing”+”or”-“button

6.Press “STOP” and “Du Du” sound would be heard

7Press “MODE” to “M” lamp is light

8.Adjust the data to +2 by pressing”+”or”-“button

9.Press “STOP” and “Du Du” sound would be heard

10.Press”FRAME” to “P” lamp is light

11Adjust the data to +2 by pressing”+”or”-“button

12Press “STOP” and “Du Du” sound would be heard

13 Switch off the power

E.REMAINS SETTING UP AFTER ROUGHING GRINDING

1.Press”L” and switch on the power.”Du Du”sound would be heard

2.Adjust ”CR” FRAME lamp to be light

3.Press”+”to +0.30,

4.Press “STOP” and “Du Du” sound would be heard

5.”GL”FRAME lamp is light.

6.Press ”+” to +0.60

7.Press”STOP” and “Du Du”sound would be heard.

8.Press”MODE” until rimless lamp is light.

9.Press”+” to +0.50.

10.Press”STOP”and “Du Du”sound would be heard.

11.switch off the power.

F.DATA ADJUSTMENT IN THE MEMORY

1.Press”V” and switch on the power,”Du Du”sound would be heard.

2.Press”MODE” to adjust the lamp in the middle to be light

3.Press”+”to +1.80.

4.Press”STOP”and “Du Du”sound would be heard.

5.Press”MODE”to rimless lamp to be light.

6.Press”-“to -1.20.

7.Press”STOP”and “Du Du” sound would be heard.

8 Switch off the power.

9.Press”CHUCK”     and switch on the power,”Du Du” sound would be heard.

10.Press”MODE” to adjust until frame lamp is light.

11.Press ”+” to +1.00.

12,Press”STOP”and “Du Du”sound would be heard.

13,Press”MODE”to adjust until rimless lamp is light.

14.Press”+”to +2.00.

15 Press “STOP”and “Du Du”sound would be heard

16.switch off the power.

17 Press “MODE” and switch on the power “Du Du”sound would be heard.

18 Press “MODE”until the three lamps are light.

19 Press”STOP”and “Du Du” sound would be heard.

20.Switch off the power.